MANTHINK  /  INDUSTRIAL IoT

GD61x

LoRaWAN Gateway

Standardized Device Manual

| VERSION 1.4 | DOCUMENT Device Manual |
| --- | --- |

Document version: 1.4

Applicable models: GD61x series (GD614 and GD618 band variants)

# Product overview

GD61x is an indoor LoRaWAN gateway based on the ESP32-S3, with a Semtech SX1301 or SX1302 concentrator selected by the ordering code. It combines Ethernet, Wi-Fi and optional 4G backhaul and accepts Type-C or 5–24 V DC power. A 4G option code of G adds the 4G and PoE modules. A Web console provides network, LoRaWAN server, radio, logging and firmware-maintenance configuration.

The only external antenna is the LoRa antenna. The Wi-Fi antenna and, on configurations with 4G, the 4G antenna are installed inside the enclosure, making GD61x suitable for offices, buildings, campuses, equipment rooms and light-industrial indoor sites.

Figure  GD61x front view with LoRa, REC and SIM interfaces

## Product highlights

ESP32-S3 controller with 8 MB Flash and 8 MB PSRAM.

Semtech SX1301 or SX1302 concentrator selected by the model suffix.

A LoRa architecture: one LoRa antenna, 8 frequency points, 65 channels and half-duplex operation.

GD614 is the 400 MHz-class RF version; GD618 is the 800 MHz-and-above RF version. The model number does not indicate channel count.

Ethernet and Wi-Fi STA backhaul; a 4G option code of G adds 4G and PoE. Ethernet is preferred by default, with automatic failover.

Wi-Fi Soft-AP for local configuration and recovery access.

Semtech UDP Packet Forwarder, Basic Station and MQTT Bridge.

DHCP, static IPv4, TLS certificates and a separate remote MQTT configuration channel; configurations with 4G support APN and SIM PIN settings.

A/B OTA partitions, runtime logs, filesystem management and factory reset.

External LoRa antenna; internal Wi-Fi antenna and internal 4G antenna when 4G is fitted.

Type-C and 5–24 V DC input on all variants; PoE when the 4G option code is G.

## Technical specifications

| Category | Parameter | Specification |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MODELS AND COMPUTE |  |  |  |  |  |
|  | Product family | GD61x |  |  |  |
|  | Model-code format | `GD61{4\ | 8}-A{N\ | G}-{regional-standard}-{version-code}-{1\ | 2}` |
|  | Band variants | GD614 and GD618 |  |  |  |
|  | 4G / PoE option | In `-A{N\ | G}, N means no 4G or PoE; G` means 4G and PoE are fitted |  |  |
|  | Version code | N: default version; other values: customer-specific or special versions |  |  |  |
|  | Controller | ESP32-S3 (ESP32-S3-WROOM-1 N8R8) |  |  |  |
|  | Flash | 8 MB, QIO, 80 MHz |  |  |  |
|  | PSRAM | 8 MB Octal PSRAM, 80 MHz |  |  |  |
|  | Configuration storage | SPIFFS; settings persist through power loss |  |  |  |

| Category | Parameter | Specification |
| --- | --- | --- |
| LORAWAN |  |  |
|  | Concentrator | Model suffix 1: Semtech SX1301; model suffix 2: Semtech SX1302 |
|  | LoRa architecture | A: one LoRa antenna, 8 frequency points, 65 channels, half-duplex |
|  | Model and RF range | GD614: 400 MHz class; GD618: 800 MHz and above; the model number does not indicate channel count |
|  | Channel bandwidth | Multi-SF channels default to 125 kHz; the LoRa Std channel is configurable |
|  | Regional plans | GD614: CN470 and EU433; GD618: EU868, AS923, AU915 and US902; the ordered RF hardware and local regulations must match |
|  | Uplink protocols | Semtech UDP Packet Forwarder, Basic Station and MQTT Bridge |
|  | Basic Station | Direct LNS or CUPS bootstrap; TLS, token or client certificate |
|  | Antenna | One external LoRa antenna connector; supplied omnidirectional antenna depends on configuration |
|  | TX power/sensitivity | Determined by the ordered regional RF front end and antenna; no single value is declared in this guide |

| Category | Parameter | Specification |
| --- | --- | --- |
| NETWORK AND BACKHAUL |  |  |
|  | Ethernet | W5500, 10/100 Mbps, RJ45 |
|  | Ethernet addressing | DHCP or static IPv4 with gateway and primary/secondary DNS |
|  | Wi-Fi | ESP32-S3 integrated 2.4 GHz Wi-Fi, AP + STA |
|  | Default Wi-Fi AP | SSID: LGW-Config; password: lgw12345678; management address: 192.168.4.1 |
|  | 4G option | Fitted when the 4G option code is G; omitted when it is N |
|  | 4G interface | miniPCIe on configurations with 4G; the modem can be replaced for different operator bands |
|  | Default 4G class | LTE Cat 1 on configurations with 4G; default modem bands match GDI51 |
|  | LTE-FDD bands | Default modem with 4G: B1 / B3 / B5 / B8 |
|  | LTE-TDD bands | Default modem with 4G: B34 / B38 / B39 / B40 / B41 |
|  | SIM | External SIM slot on configurations with 4G |
|  | Link selection | Automatic Ethernet/Wi-Fi STA failover; configurations with 4G add cellular backhaul; Ethernet is preferred by default |

| Category | Parameter | Specification |
| --- | --- | --- |
| INTERFACES |  |  |
|  | LoRa | External antenna connector |
|  | SIM | Front SIM slot for configurations with 4G |
|  | REC (Recovery) | Factory-reset button |
|  | Type-C | 5 V power and local maintenance |
|  | RJ45 | Ethernet data on all variants; PoE when the 4G option code is G |
|  | Terminal block | 5–24V, G, A+, B- |
|  | RS-485 | A+ and B- are reserved and have no user function in the current release |

| Category | Parameter | Specification |
| --- | --- | --- |
| POWER |  |  |
|  | Type-C | 5 V DC |
|  | DC terminal | 5–24 V DC |
|  | PoE | Fitted when the 4G option code is G; use PoE equipment matched to the delivered hardware |
|  | Multiple power inputs | Do not connect more than one input unless project-specific delivery documents explicitly allow it |

| Category | Parameter | Specification |
| --- | --- | --- |
| SOFTWARE AND MAINTENANCE |  |  |
|  | Management | Chinese/English Web console, REST API and separate remote MQTT configuration channel |
|  | Network time | SNTP with multiple-source retry |
|  | Firmware update | Web OTA with A/B application partitions |
|  | Logs | Live logs, level/component filters, pause, clear and download |
|  | User security | Password login, token authentication and first-login password-change reminder |
|  | Factory reset | Restores network and gateway settings and resets the login password; preserves the programmed Gateway EUI |

| Category | Parameter | Specification |
| --- | --- | --- |
| ENCLOSURE AND INSTALLATION |  |  |
|  | Environment | Indoor |
|  | Mounting | Desktop or wall mounting with the rotating bottom bracket |
|  | External antenna | LoRa only |
|  | Internal antennas | Wi-Fi and, when fitted, 4G |
|  | Dimensions, weight, ingress rating and temperature range | Not declared in this revision; use the order or delivery specification for project selection |

# Before you begin

This guide covers GD61x model identification, interfaces, installation, power-up and hardware precautions. GD614 and GD618 use the same GD6 configuration system; see the GD6 Configuration Guide for Web login, network backhaul, LoRaWAN server, radio and system-maintenance tasks. The 4/8 distinction identifies the LoRa RF range, not the channel count: GD614 is the 400 MHz-class version, while GD618 is the 800 MHz-and-above version.

Prepare the following:

A LoRa antenna matching the ordered RF band.

Type-C 5 V or 5–24 V DC terminal power; a 4G option code of G can also use compatible PoE.

Ethernet or a Wi-Fi network; an activated SIM is also required when the 4G option code is G and 4G backhaul is used.

A computer or phone able to reach the gateway's local address.

NOTE Important: Attach the LoRa antenna before applying power. Do not connect multiple power sources at once, do not apply power to the reserved A+/B- pins, and do not remove power during OTA or factory reset.

# Identify the model and device

## Identify the complete model code

### Model example

GD618-AG-AS923-N-2

### Model component breakdown

GD618: product series and RF band.

GD61 identifies a sixth-generation indoor LoRaWAN gateway.

8 identifies the 800 MHz-and-above RF version for EU868, AS923, AU915 and US902.

Other band option: 4 identifies the 400 MHz RF version for EU433 and CN470.

-AG: LoRa architecture and 4G / PoE configuration.

A identifies one LoRa antenna, 8 frequency points, 65 channels and half-duplex operation.

G identifies a configuration with both 4G and PoE modules.

Other 4G / PoE option: N identifies a configuration without 4G or PoE modules.

-AS923: LoRaWAN regional standard.

AS923 means that the gateway supports the LoRaWAN AS923 regional parameters.

Other regional codes are CN470, EU433, EU868, AU915 and US902; US902 denotes the LoRaWAN US915 (902–928 MHz) regional parameters.

The regional code must match the RF version, antenna and local regulations.

-N: default version.

N identifies the default version.

Other values identify customer-specific or special versions.

-2: LoRa concentrator chipset.

2 identifies the Semtech SX1302.

Other chipset option: 1 identifies the Semtech SX1301.

### Complete model meaning

The complete meaning of GD618-AG-AS923-N-2 is:

This is a sixth-generation indoor LoRaWAN gateway using the 800 MHz-and-above RF version for AS923, with one LoRa antenna, 8 frequency points, 65 channels and half-duplex operation. It includes the 4G and PoE modules, uses the LoRaWAN AS923 regional parameters, identifies the default version with N, and uses a Semtech SX1302 concentrator.

The front panel contains the LoRa connector, REC (Recovery) factory-reset button and SIM slot. Only LoRa uses an external antenna. The Wi-Fi antenna and the 4G antenna on G configurations are internal.

Figure  GD61x rear interfaces

| Interface | Purpose |
| --- | --- |
| USB | Type-C 5 V input and local servicing |
| 5–24V / G | Positive wide-range DC input / ground |
| A+ / B- | Reserved RS-485 pins; no operational device should be connected in this release |
| POE | 10/100 Mbps Ethernet on all configurations; PoE input only on G configurations |

The rotating bottom bracket supports wall mounting. Keep the bottom ventilation openings clear.

# Installation and power-up

## Attach the LoRa antenna

Verify that the antenna band matches the gateway RF version.

Hand-tighten the antenna on the LoRa connector without side-loading it.

Never start LoRa transmission with the antenna disconnected.

## Install a SIM

This section applies only to G configurations. For 4G backhaul, power the gateway off and insert an activated SIM in the SIM slot. A SIM without a PIN can dial directly. If PIN protection is enabled, enter its 4–8 digit PIN in the Web console.

## Select one power method

Use exactly one of these methods:

Type-C: connect a stable 5 V supply.

DC terminal: connect the positive lead of a 5–24 V DC supply to 5–24V and negative to G.

PoE (G configurations only): connect a compatible powered Ethernet cable to POE.

After power-up, the LED slow-blinks continuously (1 s on / 1 s off by default). System initialization, IP acquisition and LoRa service startup take some time.

# Configure the GD6 system

GD61x uses the shared GD6 configuration system. The following tasks are maintained in the GD6 Configuration Guide:

Web-console access and sign-in.

Ethernet, Wi-Fi and optional 4G backhaul.

Semtech UDP, MQTT Bridge and Basic Station.

LoRaWAN region, sub-band and channel settings.

Logs, firmware, filesystem, users and remote configuration.

Post-reset configuration state and common troubleshooting.

The ordering code and technical specifications in this guide remain authoritative for the hardware capabilities of a particular GD61x unit.

# REC (Recovery) button and factory reset

REC has one function only: factory reset. It does not enable the Wi-Fi AP or perform any other operation. Hold REC; the gateway follows this sequence: solid at the 2 s threshold → reset runs → three fast flashes → reboot.

Factory reset restores network, LoRaWAN, MQTT, remote settings and, on G configurations, 4G settings. It changes the login password back to wifi!0804, preserves the programmed Gateway EUI and does not overwrite the Web static files.

# Indicator patterns

| ID | Pattern | Meaning |
| --- | --- | --- |
| FR6 | LED slow-blinks continuously (1 s on / 1 s off by default) | Normal operation |
| FR7 | Factory reset: solid at the 2 s threshold → reset runs → three fast flashes → reboot | Factory reset |

# Troubleshooting

| Symptom | Checks and recovery |
| --- | --- |
| 4G on a G configuration will not register | Check SIM, PIN, APN, signal and modem bands; verify internal antenna and miniPCIe modem installation |
| No LoRa uplink | Check antenna, regional band, Gateway EUI, LNS address/ports and active backhaul |
| Gateway does not return after OTA | Allow the automatic reboot to finish; if still unavailable, recover through the Wi-Fi AP or a controlled serial service procedure instead of repeated power cuts |
| Web, login, Ethernet or Wi-Fi configuration problem | Follow Configuration troubleshooting in the GD6 Configuration Guide |

# Related documents

GD6 Configuration Guide

LoRaWAN Gateway documentation index