MANTHINK  /  INDUSTRIAL IoT

GDO51

LoRaWAN Gateway

Standardized Device Manual

| VERSION 2.01 | DOCUMENT Device Manual |
| --- | --- |

# Product Description

GDO51 is an outdoor multi-channel high-performance gateway designed by ManThink for LoRaWAN systems. It is not only suitable for the construction of various low-power networks such as cities, factories, and parks, but also suitable for data collection and transmission in harsh environments such as deserts, grasslands, and Gobi. Due to its strong processing power and flexible secondary development environment, it can also play an important role in the next generation of technology waves such as edge computing, AI, and big data.

# Main features

Supports built-in TKL (including LoRaWAN NS and IoT platform)

Supports historical data storage

Supports Thing Model

Supports Dashboard

IP67 protection level

Support POE, Type-C power supply

Support 4G, WIFI, Ethernet

8 frequency bands (maximum support 16 frequency points)

maxium support 32(default 16) demodulators capable of processing 32 packets simultaneously

(16 × SF5-SF12 LoRa®demodulators; 16 × SF5-SF10 LoRa®demodulators)

maxium 130 (default 65) channels (each channel configuration includes distinct SF, BW, and frequency points; BW = 125 kHz, totaling 65 channels)

Receiver sensitivity: -141 dBm (SF = 12, BW = 125 kHz)

Maximum RF transmit power: 22 dBm

Support US902, AU915, AS923, EU868, EU433, CN470 LoRaWAN standard

External SIM card and debugging interface for easy maintenance and repair

Seamless connection to LoRaWAN netservers such as TTN, ChirpStack, and Basicstaion

Supports remote maintenance and upgrades

Web-based visualization and friendly configuration

# Product Parameters

# Electrical parameters

| Category | Parameter | Specification |
| --- | --- | --- |
| COMPUTE AND STORAGE |  |  |
|  | SOC | LS1012A 64-bit ARM Cortex-A53 800MHz 2W |
|  | Memory type/rate | Support DDR3L 1066 MHz |
|  | Memory capacity | Memory:512MB |
|  | Storage | 8GB EMMC |

| Category | Parameter | Specification |
| --- | --- | --- |
| SOFTWARE |  |  |
|  | Platform | Built-in NS and IoT platform (with integrated TKL) |
|  | Protocol/platform support | ThinkLink, ChirpStack, TTN, Basic Station, and GWMP protocols |
|  | Databases | PostgreSQL and Redis |
|  | Historical data | historical data storage |
|  | Leaf mode | Leaf mode |
|  | Thing Model | Thing Model |
|  | Application data parsing | application data parsing |
|  | Visualization | Dashboard |

| Category | Parameter | Specification |
| --- | --- | --- |
| LORA |  |  |
|  | Operating frequency bands | CN470: 470–510 MHz EU868: 863–870 MHz EU433: 433–434 MHz AS923: 920–928 MHz US902: 902–928 MHz AU915: 915–928 MHz |
|  | Communication rate | 292bps to 15.625kbps |
|  | Spreading factors | supports SF5 to SF12 |
|  | Transmit power | 22dBm (Max) |
|  | Receiver sensitivity | -141 dBm (SF = 12, BW = 125 kHz) |
|  | Antenna type | Omnidirectional |
|  | Frequency points | 8 (maximum 16)(BW = 125 kHz) |
|  | Demodulators | 16 (maximum 32) |
|  | Concurrent packets | 16 (maximum 32) packets simultaneously |
|  | Demodulator composition | 8(maximum 16) × SF5-SF12 LoRa®demodulators; 8 (maximum 16) × SF5-SF10 LoRa®demodulators |
|  | Channels | 65(maximum 130) |
|  | Channel definition/note | each channel is defined by a unique combination of SF, BW, and frequency point; BW = 125 kHz, totaling 65 channels |
|  | Operating mode | Half-duplex |
|  | Single-channel bandwidth | 125KHz or higher |

| Category | Parameter | Specification |
| --- | --- | --- |
| NETWORK AND BACKHAUL |  |  |
|  | Ethernet | 1000M/100M |
|  | 4G uplink | The gateway uses CAT1 module for 4G uplink by default |
|  | LTE-FDD bands | B1/3/5/8 |
|  | LTE-TDD bands | B34/38/39/40/41 |
|  | 4G deployment note | If the operator's frequency band in the planned deployment area is not within the above, please contact ManThink technical personnel to replace the appropriate 4G module. |

| Category | Parameter | Specification |
| --- | --- | --- |
| PORTS AND TIMING |  |  |
|  | USB Type-C configuration | USB-TypeC |
|  | Ethernet configuration | Ethernet |
|  | Wi-Fi configuration | WiFi |
|  | SIM card interface | nano SIM external interface (no need to open the cover for replacement) |
|  | Timing | GPS |

| Category | Parameter | Specification |
| --- | --- | --- |
| POWER |  |  |
|  | Power supply | POE(48V) |
|  | Power consumption | 5W |

| Category | Parameter | Specification |
| --- | --- | --- |
| ENVIRONMENT AND ENCLOSURE |  |  |
|  | Operating temperature | -40℃～70℃ |
|  | Operating humidity | 0%～95%RH |
|  | Water and dust resistance | IP67 |
|  | Weight | 1.2KG |
|  | Dimensions | 164x124x50mm |
|  | Installation method | Pole or wall mounting |

# Accessories List

Figure  Accessories List

# Product Dimensions

Figure  Product Dimensions

# Interface Description

| Interface Name | Description |
| --- | --- |
| RF | LoRa Antenna Interface |
| WIFI | WIFI Antenna Interface |
| POE | POE Power Interface |
| SIM | SIM Card Slot |
| 4G | 4G Antenna Interface |
| GPS | GPS Antenna Interface |

# Indicator Light Description

| Indicator | Color / network | Light states and meanings |
| --- | --- | --- |
| Running indicator | — | Steady on: The system is starting. Off: Basic system startup is complete and application processes are starting. Flashing at 1 Hz: The system is running normally. |
| Network indicator | Blue/wired network Green/4G network | Blue, flashing at 1 Hz: The wired network connection is normal. Blue, flashing every 500 ms: The wired network connection is abnormal. Blue, flashing rapidly: Network configuration is in progress. Green, off: No 4G module is installed. Green, steady on: A 4G module is installed, but no SIM card is inserted. Green, flashing irregularly: Data is being exchanged over the 4G network. |
| LoRa data transceiver indicator | Green Blue | Green, flashing: A LoRa data packet is being transmitted. Blue, flashing: A LoRa data packet is being received. |
| LAN status light | Red Blue | Red: System control status. Blue: Wi-Fi connection status. |
| Fault indicator | — | Flashing rapidly: The system is in the second startup phase. Off: No system fault is present. Flashing: The system has detected a fault. |

# Product Model

## Model example

GDO518-AG-AS923-N-2

## Model component breakdown

GDO518: product series and RF band.

GDO51 identifies a fifth-generation outdoor LoRaWAN gateway.

8 identifies the 800 MHz-and-above RF version for EU868, AS923, AU915 and US902.

Other band option: 4 identifies the 400 MHz RF version for EU433 and CN470.

-AG: LoRa architecture and 4G configuration.

A identifies one LoRa antenna, 8 frequency points, 65 channels and half-duplex operation.

Other LoRa architecture option: D identifies two LoRa antennas, 16 frequency points, 130 channels and full-duplex operation.

G identifies an integrated 4G module.

Other 4G option: N identifies a configuration without a 4G module.

-AS923: LoRaWAN regional standard.

AS923 means that the gateway supports the LoRaWAN AS923 regional parameters.

Other regional codes are CN470, EU433, EU868, AU915 and US902; US902 denotes the LoRaWAN US915 (902–928 MHz) regional parameters.

-N: default version.

N identifies the default version.

Other values are reserved for customized or special versions.

-2: LoRa concentrator chipset.

2 identifies the Semtech SX1302.

Other chipset option: 1 identifies the Semtech SX1301.

## Complete model meaning

The complete meaning of GDO518-AG-AS923-N-2 is:

This is a fifth-generation outdoor LoRaWAN gateway in the 800 MHz-and-above RF class. It uses one LoRa antenna, 8 frequency points, 65 channels and half-duplex operation, includes a 4G module, supports the LoRaWAN AS923 regional standard, uses the default version code N, and is equipped with a Semtech SX1302 concentrator.

# Gateway Installation

Note: It is not necessary to disassemble the Gateway shell during installation.

# Installation Direction

Figure  Installation Direction

Note: The front view of the product is shown in the picture above. GPS and 4G antenna interface to the right, WiFi antenna interface down. During the installation process, please ensure that the product orientation is consistent with the above figure for correct installation.

# Backplane Installation

Flip the gateway to the direction shown in the figure and confirm that the backplane is tightly connected to the Gateway.

Align the screw with the screw hole and use a screwdriver to turn it clockwise until the screw cannot turn.

Figure  Backplane Installation

# Network cable installation

Thread the waterproof plug into the network cable from right to left in the following order, and assemble parts 1, 2, 3 and 4 tightly

Insert one end of the network cable crystal head into the Gateway POE interface

Move the assembled waterproof plug part to the direction of the Crystal Head, and screw it on the POE interface to ensure that the assembled waterproof plug is tightly connected with the POE interface.

Figure  Network cable installation

Figure  Network cable installation

# SIM card installation

Unscrewing the waterproof cover on the SIM interface

Keep the SIM card metal patch position towards the TYPE-C interface

Insert the SIM card into the card slot and tighten the waterproof cover.

Figure  SIM card installation

# Power connection

OUT port connection gateway POE interface (power supply)

The IN port is connected to a switch or router (network supply). If there is 4G, there is no need to connect the IN interface.

Figure  Power connection

# FRP antenna installation

Connect the LoRa antenna to the Gateway RF interface

Note: This step is required for the optional lora antenna.

Figure  FRP antenna installation

# Gateway fixed

## Wall-mounted fixed

Install with the Gateway face up (Figure 1)

Four holes corresponding to the holes of the back plate bracket are made in the wall, and then the expansion bolts are driven in (the positions of the four holes of the back plate bracket can be marked on the wall first)

Pass the expansion screw through the back bracket and screw in the expansion screw with a tool.

Figure  Wall-mounted fixed

## Holding Rod fixing

Install with the Gateway face up (Figure 2)

Confirm that the pole diameter is between 30mm-52mm

Combine the holding Rod clamps according to the position shown in the figure, and tighten the screws.

Figure  Holding Rod fixing

# Precautions

# Waterproof protection

Use insulating tape or waterproof material to wrap and seal the connection to ensure that it is not penetrated by moisture and moisture.

Pay special attention to areas where moisture can easily enter, such as outdoor installations or areas exposed to rain or high humidity.

The insulating tape is tightly wrapped around the connection, completely covered, and forms a waterproof barrier.

Check the waterproof condition regularly, and re-coat or replace the insulating tape as needed to maintain effective protection.

Ensure that all connections and cables are properly sealed and protected to prevent moisture damage and maintain the integrity of the installation.

# Gateway waterproof

Ensure all cable entry points and connectors on the Gateway are properly sealed following the installation steps

Pay particular attention to areas where moisture may enter, such as cable connections, antenna interfaces, and power input ports.

Apply a layer of waterproof material around the cable entry point and connector, completely covering to prevent moisture penetration.

Check the waterproof condition regularly, and re-coat or replace the waterproof material as needed to maintain effective protection.

# POE adapter waterproof installation

Cose a suitable location to install the POE Adapter, making sure it is away from direct water sources or areas that may splash water.

Make sure that the POE Adapter is installed vertically with the connection port down to prevent water from entering.

Use a waterproof material, such as insulating tape or waterproof casing, to cover and seal the connection of the POE Adapter.

Pay attention to the entry points of the cables and make sure they are properly sealed to prevent water from entering.

Check waterproofing measures regularly and re-coat or replace any damaged or aged materials.

Please note that waterproofing requirements may vary depending on the specific environment and installation conditions. To ensure effective waterproofing, follow best practices and consult a professional if needed.

# User Guide

# Gateway Configuration

AN-25100108 Gateway User Guide

# Using ThinkLink

\ThinkLink User Guide

# Connecting to Chirp Stack

GW-AN08: Connecting to ChirpStack through Basic Station LNS (with TLS)

# Connecting to TTN

GW-AN09: Connecting to TTN through Basic Station

# Contact Us

Website: www.manthink.cn

Email: info@manthink.cn

Tel: +86-15810684257